SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Code of Conduct:

I Pakshalika SV/ 1912260154 (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: Pakshalika SV

| Criteria | Weightage | Q1 10marks | Q2 10marks | Q3 10marks | Q4 10marks |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% |  |  |  |  |
| Method Selection | 20% |  |  |  |  |
| Solution Process | 25% |  |  |  |  |
| Accuracy of Calculation | 20% |  |  |  |  |
| Interpretation of Results | 15% |  |  |  |  |

Total Marks 		/40						Signature of the Faculty

Annexure A

ANALYTICAL PROBLEM SOLVING

1. A university network is assigned the IP address block 192.168.20.0/24 for its internal communication. The network administrator has decided to divide this network into subnets for three different labs using fixed subnet masks. Lab A is assigned a /26 subnet, Lab B is assigned a /27 subnet, and Lab C is assigned a /28 subnet. Based on the given subnet masks, apply subnetting concepts to determine the network address of each lab. Further, identify the valid range of host IP addresses and the broadcast address for each subnet. Finally, calculate the total number of usable host IP addresses available in each lab’s subnet.

2. A network administrator has configured two systems with the following details: System 1 has IP address 192.168.1.130 with subnet mask 255.255.255.192, and System 2 has IP address 192.168.1.190 with subnet mask 255.255.255.192. Using the given configuration, apply subnetting techniques to determine the subnet range, network address, and broadcast address for each system. Identify the valid host IP range for both subnets based on the given subnet mask. Calculate whether each IP address falls within its respective subnet range. Also, determine the number of usable host addresses available in each subnet.

3. An organization has been allocated the IP block 172.16.0.0/16 for its internal network. The network administrator has already decided the subnet masks for each department as follows: HR → /25, Sales → /26, IT → /27, and Admin → /28. Using the given subnet masks, apply subnetting techniques to assign suitable subnet addresses for each department. Determine the network address, valid host IP range, and broadcast address for each subnet. Based on the allocation, calculate the number of usable host addresses in each department. Finally, identify the remaining unused IP address range within the given network block.

4. A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a /27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

5. A network uses the Internet Protocol version 6 address 2001:0db8:0000:0000:0000:ff00:0042:8329. Apply IPv6 addressing rules to compress the given address into its shortest form and then expand the compressed address back to its full notation. Using a prefix length of /64, determine the network prefix of the given address. Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |

ANSWERS – ANALYTICAL PROBLEM SOLVING

Question 1: Subnetting the university network 192.168.20.0/24

Given: Address block = 192.168.20.0/24. Lab A requires /26, Lab B requires /27, and Lab C requires /28.

Formulae used: Total addresses = 2^(32 − prefix length); usable hosts = total addresses − 2. The first address is the network address and the last address is the broadcast address.

A /26 subnet has 64 addresses and block size 64; a /27 subnet has 32 addresses and block size 32; a /28 subnet has 16 addresses and block size 16.

| Lab | Prefix / Subnet mask | Network address | Valid host range | Broadcast address | Usable hosts |
| --- | --- | --- | --- | --- | --- |
| Lab A | /26 — 255.255.255.192 | 192.168.20.0 | 192.168.20.1 – 192.168.20.62 | 192.168.20.63 | 62 |
| Lab B | /27 — 255.255.255.224 | 192.168.20.64 | 192.168.20.65 – 192.168.20.94 | 192.168.20.95 | 30 |
| Lab C | /28 — 255.255.255.240 | 192.168.20.96 | 192.168.20.97 – 192.168.20.110 | 192.168.20.111 | 14 |

Working: The subnets are allocated consecutively without overlap. Lab A occupies .0–.63, Lab B occupies .64–.95, and Lab C occupies .96–.111, all within 192.168.20.0/24.

Result: Lab A has 62 usable host addresses, Lab B has 30, and Lab C has 14.

Question 2: Determine the subnets for two systems

Given: System 1 = 192.168.1.130; System 2 = 192.168.1.190; subnet mask = 255.255.255.192 = /26.

Formulae used: Addresses per subnet = 2^(32 − 26) = 64; usable hosts = 64 − 2 = 62. Block size = 256 − 192 = 64.

The /26 subnet boundaries in the last octet are 0, 64, 128, and 192. Both .130 and .190 fall in the 128–191 block.

| Item | System 1 | System 2 |
| --- | --- | --- |
| IP address | 192.168.1.130 | 192.168.1.190 |
| Subnet mask | 255.255.255.192 (/26) | 255.255.255.192 (/26) |
| Subnet address range | 192.168.1.128 – 192.168.1.191 | 192.168.1.128 – 192.168.1.191 |
| Network address | 192.168.1.128 | 192.168.1.128 |
| Valid host range | 192.168.1.129 – 192.168.1.190 | 192.168.1.129 – 192.168.1.190 |
| Broadcast address | 192.168.1.191 | 192.168.1.191 |
| Is the given IP a valid host? | Yes | Yes |
| Usable host addresses | 62 | 62 |

Verification: Both IP addresses lie in the valid host range 192.168.1.129–192.168.1.190. Hence, they belong to the same subnet, 192.168.1.128/26.

Result: Both systems are in the same subnet, which provides 62 usable host addresses.

Question 3: Departmental subnet allocation from 172.16.0.0/16

Given: Parent network = 172.16.0.0/16. HR uses /25, Sales /26, IT /27, and Admin /28.

Formulae used: Total addresses = 2^(32 − prefix length); usable hosts = total addresses − 2. Subnets are allocated consecutively with correct block alignment.

The /16 network contains 65,536 addresses. The required subnet sizes are 128, 64, 32, and 16 addresses respectively.

| Department | Prefix / Mask | Network address | Valid host range | Broadcast address | Usable hosts |
| --- | --- | --- | --- | --- | --- |
| HR | /25 — 255.255.255.128 | 172.16.0.0 | 172.16.0.1 – 172.16.0.126 | 172.16.0.127 | 126 |
| Sales | /26 — 255.255.255.192 | 172.16.0.128 | 172.16.0.129 – 172.16.0.190 | 172.16.0.191 | 62 |
| IT | /27 — 255.255.255.224 | 172.16.0.192 | 172.16.0.193 – 172.16.0.222 | 172.16.0.223 | 30 |
| Admin | /28 — 255.255.255.240 | 172.16.0.224 | 172.16.0.225 – 172.16.0.238 | 172.16.0.239 | 14 |

Working: The allocations use 128 + 64 + 32 + 16 = 240 addresses, from 172.16.0.0 through 172.16.0.239.

Remaining unused range: 172.16.0.240 through 172.16.255.255. This is a continuous range within the original /16 block, although not a single CIDR subnet.

The unused range contains 65,296 addresses (65,536 − 240). Result: HR has 126 usable hosts, Sales 62, IT 30, and Admin 14.

Question 4: Subnetting 172.16.10.0/24 using /27

Given: Parent network = 172.16.10.0/24; new prefix = /27; target IPs = 172.16.10.78 and 172.16.10.95.

Formulae used: Number of subnets = 2^(27 − 24) = 8. Addresses per /27 subnet = 2^(32 − 27) = 32; usable hosts = 32 − 2 = 30.

The /27 mask is 255.255.255.224, giving block size 256 − 224 = 32. The subnet boundaries are .0, .32, .64, .96, .128, .160, .192, and .224.

| Subnet | Network address | Valid host range | Broadcast address |
| --- | --- | --- | --- |
| 1 | 172.16.10.0/27 | 172.16.10.1 – 172.16.10.30 | 172.16.10.31 |
| 2 | 172.16.10.32/27 | 172.16.10.33 – 172.16.10.62 | 172.16.10.63 |
| 3 | 172.16.10.64/27 | 172.16.10.65 – 172.16.10.94 | 172.16.10.95 |
| 4 | 172.16.10.96/27 | 172.16.10.97 – 172.16.10.126 | 172.16.10.127 |
| 5 | 172.16.10.128/27 | 172.16.10.129 – 172.16.10.158 | 172.16.10.159 |
| 6 | 172.16.10.160/27 | 172.16.10.161 – 172.16.10.190 | 172.16.10.191 |
| 7 | 172.16.10.192/27 | 172.16.10.193 – 172.16.10.222 | 172.16.10.223 |
| 8 | 172.16.10.224/27 | 172.16.10.225 – 172.16.10.254 | 172.16.10.255 |

Subnet containing 172.16.10.78: It falls between .64 and .95, so it belongs to 172.16.10.64/27. Network address = 172.16.10.64; valid host range = 172.16.10.65–172.16.10.94; broadcast address = 172.16.10.95.

Does 172.16.10.95 belong to the same subnet? It is inside the subnet's address block but is the broadcast address, not a valid host address. It cannot be assigned to a host.

Result: Eight subnets are created, each with 30 usable host addresses. The IP .78 is a valid host in subnet 3, while .95 is that subnet's broadcast address.

Question 5: IPv6 address compression, expansion, and /64 prefix

Given IPv6 address: 2001:0db8:0000:0000:0000:ff00:0042:8329; prefix length = /64.

Rules used: Remove leading zeros from each hextet, then replace one consecutive run of all-zero hextets with ::. A /64 prefix is the first four hextets (64 bits).

Step 1 – Compress: Removing leading zeros gives 2001:db8:0:0:0:ff00:42:8329. Compress the three consecutive zero hextets.

Shortest form: 2001:db8::ff00:42:8329

Step 2 – Expand: Three zero hextets are inserted by :: to restore eight hextets.

Expanded form: 2001:0db8:0000:0000:0000:ff00:0042:8329

Step 3 – Network prefix: The first four hextets form the 64-bit prefix.

Network prefix: 2001:0db8:0000:0000::/64 (equivalently, 2001:db8:0:0::/64).

Step 4 – Possible interface identifiers: 128 − 64 = 64 bits remain for the interface identifier. Therefore, the number of possible 64-bit values is 2^64 = 18,446,744,073,709,551,616.

Unlike conventional IPv4 subnet calculations, IPv6 does not subtract network and broadcast addresses in the same way. Thus, 2^64 is the number of possible interface-identifier values in the /64.

Result: Compressed address = 2001:db8::ff00:42:8329; expanded address = 2001:0db8:0000:0000:0000:ff00:0042:8329; prefix = 2001:db8:0:0::/64; interface identifier space = 2^64.